Superposition of Waves Using Audacity

Open Audacity.

Go to Generate and choose Tone...

Generate a sine wave of frequency 50 Hz and amplitude 0.5

Go to Project and Select New Audio Track.

On this track, generate a tone of frequency 150 Hz and amplitude 0.17.

(3 x the original frequency, 1/3 of the amplitude).

Repeat for a new audio track with a frequency 150 Hz and amplitude 0.1.

These are the Fourier components of a square wave. We could continue to add the odd harmonics of n times the original frequency and 1/n the original amplitude.

Go to File.

Choose Export as WAV...

If the message on the right appears, click OK.

Save your file.

You should now have mixed all your individual components together.

Go to File and Open...

Re-open the file you have just saved.

You should see something similar to the image on the right.

To examine individual waves, click the zoom button:

You could try to create beats – use two tracks where the frequencies differ by only a small amount, say 500 Hz and 498 Hz.